Technical Assistance in  the use electro mechanical pumps moved by draught animal power

BENEFITS

| WATER | a larger amount of available water because the new pumps makes possible the supply of water from waterlayers deeper than those exploited with traditional mechanical pumps |
| --- | --- |
|  | less salty water because lifted from deeper layers and then not evaporated |
|  | lifting of water not polluted by the walking of animals, dung and other biological products because the animals can work far from the well |
|  | the use of an electrical pump allows to lift water from plastic or iron tubewells and this can avoid the closure of the hole now so frequent due to the falldown of the walls of the well. |
|  | The system is ideal to create a water network with lifting water on the top of a distribution tower |
| HEALTH | Electrification of dispensaries |
|  | Electrification of health care equipment such as surgical lamps, laboratories, analyzers and refrigerator for vaccines, blood, and others perishable pharmaceutics and drugs |
| COMMUNICATION AND EDUCATION | Mobile telephone systems, PC, TV and radio sets in particular for education and weather forecasts |
|  | school and class lighting |

OUTPUT

|  | animal power in circular track (*) |  | equipment conversion capacity |  |  | total energy output |
| --- | --- | --- | --- | --- | --- | --- |
|  | Hp | W | mechanical | electrical (***) | instant power w | 4 hours work(**) wh |
| donkey | 0,16 | 118 | 0,95 | 0,75 | 84 | 335 |
| horse or light ox | 0,25 | 184 | 0,95 | 0,75 | 131 | 524 |
| ox of 400 kg | 0,32 | 235 | 0,95 | 0,75 | 168 | 671 |
| pair of small animals | 0,30 | 221 | 0,95 | 0,75 | 157 | 629 |
| pair of big animals | 0,60 | 441 | 0,95 | 0,75 | 314 | 1.258 |
| (*) elaboration from UN literature |  |  |  |  |  |  |
| (**) daily time of work for animals in circular track, divided in period of 45 minutes (time suggested by UN literature). Logically the equipment can be utilized, for more times in the day, by different animals, and this will increase the daily energy output |  |  |  |  |  |  |
| (***) direct use without batteries for pumping and similar applications |  |  |  |  |  |  |

ADVANTAGES IN THE USE OF DRAUGHT ANIMAL POWER

| connection to the electric grid | the cost |
| --- | --- |
| wind generator | the cost |
|  | shortage of area with wind speed of more the 3 meters per second (minimum speed necessary to generate electric energy) |
|  | shortage of geographical areas with continuous, constant wind and therefore a great needs of batteries for energy accumulation |
| photovoltaic | cost of the plant and of the energy accumulation |
| fuel engine (diesel, gasoline) | foreign currency cost, difficulty in fuel and spare parts supply, maintenance problems, reliability and life time of the engine |
| biogas | foreign currency cost of the engine, maintenance and spare parts supply problems |
|  | To produce enough gas for an electric generator engine it is estimated that it should be available the biological substance of at least a family and five big oxen |
| small water turbines | cost, maintenance |
|  | shortage of places with rivers with sufficient flood power |

WE DAP

Via Pieve di Cadore 30

00136 Roma I

Via San Francesco de Geronimo 59

74023 Grottaglie TA I

+ 39 063012328

+ 39 349 6934669

http://www.masseriacoppola.it/dap

info@masseriacoppola.it